|  | Staunton Harold Sailability Trust |  |
| --- | --- | --- |

Hansa Dinghy pre-sailing safety checks

These checks must be carried out prior to any sailing taking place and should be carried out each time a boat returns to the jetty for the next user.

- The weighted keel must be fully down and locked in place with the locking pin through the handle into the hole in the hull and the screw held by bungee.

- The furling mechanism must be working correctly to allow the sail to be furled according to the wind conditions and participant experience level.

- The furling line must be cleated in after adjustment to prevent the wind blowing the sail out once underway.

- Ensure the joystick turns the rudder fully to port and to starboard and that the rudder blade Is fully down.

- Ensue all ropes and pulleys are working correctly and that sails can be trimmed as necessary.

- Ensure all bungs are in and tight

Challenger Dinghy pre-sailing safety checks

These checks must be carried out prior to any sailing taking place and should be carried out each time a boat returns to the jetty for the next user.

- Checked boats for any visible damage or missing parts.

- Check all bungs are in main hull and floats.

- Check rudder mechanism is connected and steering to starboard and port has the full range.

- Check if appropriate correct positioning of seat for helm.

- Check all ropes and pulleys are working correctly and sails can be trimmed.

- Ensure if being used the buddy seat is properly positioned and secured both front and back.

Note: Please report any faults found to the session lead boats with any faults should not be used until the faults are corrected and re checked as per process.