|  | Staunton Harold Sailability Trust |  |
| --- | --- | --- |

Hereford Hoist user instructions

Safe operation requires two people

- The hoist must be located in the mounting provided on the pontoon, check it is secure and rotate the hoist arm fully to check free movement.

- Check the hanger attachment is fitted correctly on the end of the arm.

- Check the piston ensuring it is clean and free of dirt and there are no oil leaks.

- The arm is raised and lowered by hydraulic pressure, there is a valve on the cylinder that controls the speed of lowering, and a handle that pumps up the pressure raising the arm.

- To lower the arm, release the valve by turning anti clockwise till you arrive at the desired height then close by turning clockwise to finger tight.

- To raise the arm the valve must be closed then using the handle pump the arm up till it reaches the desired height, only raise it to the height required.

- To move the hoist into the correct position for lifting or lowering rotate the whole hoist by pushing using the hoist arm.

- When using the sling ensure that it is fully around the person being lifted and cross the handles before attaching to the hoist handle.

- Before lifting put tension on the sling attachments by raising the hoist arm, check by pulling that these are secure before continuing.

- Raise the person until at the desired height over the pontoon before swinging them over to the boat.

- Once in position over the seat in the boat slowly lower the arm until the person is located in the seat.

- Once in position remove the sling attachment from the hoist raise and move out of the way.

- Leave the sling in place around the person so its ready to recover them when they return.